แนวทางการใช้งานอินเทอร์เน็ตของสรรพสิ่งในระบบการผลิต IoT Approaches to Manufacturing System

ขื่อ-สกุล : วราสิริ ลิ้มประเสริฐ B6214005

5/5. คำถามท้ายบทเพื่อทดสอบความเข้าใจ

Quiz_401 – Ubidots: Monitor DHT22, Monitor Digital Switch and Control 4 LED

#include <WiFi.h> #include <PubSubClient.h> #include "DHTesp.h" const char *My_SSID = "V2036"; const char *My_Pass = "fnafchica"; const char *MQTT_Server = "things.ubidots.com"; const char *MQTT_User = "BBFF-gvcR0u8y0BegX9muo6Vfs4mKvsItpI"; const char *MQTT_Pass = "BBFF-gvcR0u8y0BegX9muo6Vfs4mKvsItpI"; const char *PTopic1 = "/v2.0/devices/bearish"; const char *STopic1 = "/v2.0/devices/bearish/humid"; const char *STopic2 = "/v2.0/devices/bearish/tempp"; const char *STopic3 = "/v2.0/devices/bearish/led1"; const char *STopic4 = "/v2.0/devices/bearish/led2"; const char *STopic5 = "/v2.0/devices/bearish/led3"; const char *STopic6 = "/v2.0/devices/bearish/led4"; const char *STopic7 = "/v2.0/devices/bearish/sw1"; const char *STopic8 = "/v2.0/devices/bearish/sw2"; #define MQTT_Port 1883 #define Test_LED1 18 #define Test_LED2 19 #define Test_LED3 22 #define Test_LED4 23 #define Test_SW1 4 #define Test_SW2 21 #define Pin_DHT22 15 DHTesp dht; WiFiClient espClient; PubSubClient client(espClient); long lastMsg = 0; char msg[50]; int value = 0; void Setup_Wifi() { delay(10); Serial.println(); Serial.print("Connecting to "); Serial.println(My_SSID); WiFi.begin(My_SSID, My_Pass); while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); } randomSeed(micros()); Serial.println(""); Serial.println("WiFi connected"); Serial.println("IP address: "); Serial.println(WiFi.localIP()); } void reconnect() { while (!client.connected()) // Loop until we're reconnected { Serial.print("Attempting MQTT connection..."); String clientId = "ESP32 Client-"; clientId += String(random(0xffff), HEX); // Create a random client ID if (client.connect(clientId.c_str(), MQTT_User, MQTT_Pass)) // Attempt to connect { Serial.println("connected"); // Once connected, publish an announcement... client.subscribe(STopic1); client.subscribe(STopic2); client.subscribe(STopic3); client.subscribe(STopic4); client.subscribe(STopic5); client.subscribe(STopic6); client.subscribe(STopic7); client.subscribe(STopic8); } else { Serial.print("failed, rc="); Serial.print(client.state()); Serial.println(" try again in 5 seconds"); delay(5000); } } } void callback(char *topic, byte *payload, unsigned int length) { Serial.print("Message arrived ["); Serial.print(topic); Serial.print("] "); for (int i = 0; i < length; i++) { Serial.print((char)payload[i]); } if (topic[24] == STopic3[24]) { Serial.print(" -LED1->> "); Serial.print((char)payload[10]); if (payload[10] == '1') digitalWrite(Test_LED1, HIGH); else digitalWrite(Test_LED1, LOW); } if (topic[24] == STopic4[24]) { Serial.print(" -LED2->> "); Serial.print((char)payload[10]); if (payload[10] == '1') digitalWrite(Test_LED2, HIGH); else digitalWrite(Test_LED2, LOW); } if (topic[24] == STopic5[24]) { Serial.print(" -LED3->> "); Serial.print((char)payload[10]); if (payload[10] == '1') digitalWrite(Test_LED3, HIGH); else digitalWrite(Test_LED3, LOW); } if (topic[24] == STopic6[24]) { Serial.print(" -LED4->> "); Serial.print((char)payload[10]); if (payload[10] == '1') digitalWrite(Test_LED4, HIGH); else digitalWrite(Test_LED4, LOW); } Serial.println(); } void setup() { pinMode(Test_LED1, OUTPUT); pinMode(Test_LED2, OUTPUT); pinMode(Test_LED3, OUTPUT); pinMode(Test_LED4, OUTPUT); pinMode(Test_SW1, INPUT_PULLDOWN); pinMode(Test_SW2, INPUT_PULLDOWN); dht.setup(Pin_DHT22, DHTesp::DHT22); Serial.begin(115200); Setup_Wifi(); client.setServer(MQTT_Server, MQTT_Port); client.setCallback(callback); } void loop() { if (!client.connected()) reconnect(); client.loop(); long now = millis(); if (now - lastMsg > 5000) { lastMsg = now; float humidity = dht.getHumidity(); float temperature = dht.getTemperature(); int sw1 = 0; int sw2 = 0; if (digitalRead(Test_SW1) == HIGH) sw1 = 1; else sw1 = 0; if (digitalRead(Test_SW2) == LOW) sw2 = 1; else sw2 = 0; snprintf (msg, 75, "{ \"humid\" : %.2f, \"tempp\": %.2f, \"sw1\": %d, \"sw2\": %d }", humidity, temperature, sw1, sw2); Serial.print("Publish message: "); Serial.println(msg); client.publish(PTopic1, msg); } }

Quiz_402 – Ubidots: Monitor DHT22 with TM1638 Display and LINE Alert

  - ส่งข้อมูลอุณหภูมิไปยัง Ubidots

  - หากอุณหภูมิที่อ่านได้เกิน 28’C ให้แจ้งเตือนผ่าน LINE และบอกด้วยว่าอุณหภูมิเท่าใด

- แสดงอุณหภูมิที่ 7_Segment Display TM1638 Board

#include <WiFi.h> #include <PubSubClient.h> #include <HTTPClient.h> #include <TM1638plus.h> #include "DHTesp.h" const char *My_SSID = "V2036"; const char *My_Pass = "fnafchica"; const char *MQTT_Server = "things.ubidots.com"; const char *MQTT_User = "BBFF-gvcR0u8y0BegX9muo6Vfs4mKvsItpI"; const char *MQTT_Pass = "BBFF-gvcR0u8y0BegX9muo6Vfs4mKvsItpI"; #define WebHooksKey "oXSQX-hS7mc2o1blAA3UlubXBXN2WIrMlIheoCkvYQI" #define WebHooksEventName "test_GSheet" #define WebHooksEventName_line "Test_Key" const char *PTopic1 = "/v2.0/devices/bearish"; const char *STopic1 = "/v2.0/devices/bearish/humid"; const char *STopic2 = "/v2.0/devices/bearish/tempp"; #define Brd_STB 18 // strobe = GPIO connected to strobe line of module #define Brd_CLK 19 // clock = GPIO connected to clock line of module #define Brd_DIO 21 // data = GPIO connected to data line of module bool high_freq = true; //default false,, If using a high freq CPU > ~100 MHZ set to true. TM1638plus tm(Brd_STB, Brd_CLK , Brd_DIO, high_freq); #define MQTT_Port 1883 #define Pin_DHT22 15 #define My_NAME "B6214005 Varasiri Limprasert" DHTesp dht; WiFiClient espClient; PubSubClient client(espClient); long lastMsg = 0; char msg[50]; int value = 0; void Setup_Wifi() { delay(10); Serial.println(); Serial.print("Connecting to "); Serial.println(My_SSID); WiFi.begin(My_SSID, My_Pass); while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); } randomSeed(micros()); Serial.println(""); Serial.println("WiFi connected"); Serial.println("IP address: "); Serial.println(WiFi.localIP()); } void reconnect() { while (!client.connected()) // Loop until we're reconnected { Serial.print("Attempting MQTT connection..."); String clientId = "ESP32 Client-"; clientId += String(random(0xffff), HEX); // Create a random client ID if (client.connect(clientId.c_str(), MQTT_User, MQTT_Pass)) // Attempt to connect { Serial.println("connected"); // Once connected, publish an announcement... client.subscribe(STopic1); client.subscribe(STopic2); } else { Serial.print("failed, rc="); Serial.print(client.state()); Serial.println(" try again in 5 seconds"); delay(5000); } } } void setup() { tm.displayBegin(); dht.setup(Pin_DHT22, DHTesp::DHT22); Serial.begin(115200); Setup_Wifi(); client.setServer(MQTT_Server, MQTT_Port); } void loop() { if (!client.connected()) reconnect(); client.loop(); long now = millis(); if (now - lastMsg > 5000) { lastMsg = now; float humidity = dht.getHumidity(); float temperature = dht.getTemperature(); snprintf (msg, 75, "{ \"humid\" : %.2f, \"tempp\": %.2f}", humidity, temperature); Serial.print("Publish message: "); Serial.println(msg); client.publish(PTopic1, msg); Serial.println(); Serial.print("\nTemperature('C) = "); Serial.print(temperature, 1); Serial.print("\tHumidity(%) = "); Serial.print(humidity, 1); String serverName = "http://maker.ifttt.com/trigger/" + String(WebHooksEventName) + "/with/key/" + String(WebHooksKey); String httpRequestData = "value1=" + String(My_NAME) + "&value2=" + String(temperature) + "&value3=" + String(humidity); Serial.println(); Serial.println("Server Name >> " + serverName); Serial.println("json httpRequestData >> " + httpRequestData); if (WiFi.status() == WL_CONNECTED) { HTTPClient http; http.begin(serverName); http.addHeader("Content-Type", "application/x-www-form-urlencoded"); int httpResponseCode = http.POST(httpRequestData); Serial.print("HTTP Response code: "); Serial.println(httpResponseCode); http.end(); if (httpResponseCode == 200) Serial.println("[Google sheet] --> Successfully sent"); else Serial.println("[Google sheet] --> Failed!"); } else { Serial.println("WiFi Disconnected"); } /// if temp > 28 C send notifications >> line if (temperature > 28) { String serverName = "http://maker.ifttt.com/trigger/" + String(WebHooksEventName_line) + "/with/key/" + String(WebHooksKey); String httpRequestData = "value1=" + String(temperature); Serial.println(); Serial.println("Server Name >> " + serverName); Serial.println("json httpRequestData >> " + httpRequestData); if (WiFi.status() == WL_CONNECTED) { HTTPClient http; http.begin(serverName); http.addHeader("Content-Type", "application/x-www-form-urlencoded"); int httpResponseCode = http.POST(httpRequestData); Serial.print("HTTP Response code: "); Serial.println(httpResponseCode); http.end(); if (httpResponseCode == 200) Serial.println("[Line] --> Successfully sent"); else Serial.println("[Line] --> Failed!"); } else { Serial.println("WiFi Disconnected"); } } /*Display */ int t = int(temperature * 100); int Tempp2 = (int)temperature / 10; int Tempp1 = (int)temperature % 10; int Tempp0 = (int)(temperature * 10) % 10; int Humi2 = (int)humidity / 10; int Humi1 = (int)humidity % 10; int Humi0 = (int)(humidity * 10) % 10; tm.displayHex(0, Tempp2); tm.displayASCIIwDot(1, Tempp1 + '0'); // turn on dot tm.displayHex(2, Tempp0); tm.display7Seg(3, B01011000); // Code=tgfedcba tm.displayHex(4, Humi2); tm.displayASCIIwDot(5, Humi1 + '0'); // turn on dot tm.displayHex(6, Humi0); tm.display7Seg(7, B01110100); // Code=tgfedcba delay(2000); int WaitTime = 10; Serial.print(" >> Wait for next time --> "); for (int i = WaitTime; i >= 0; i -= 5) { Serial.print(","); Serial.print(i); delay(5000); } } }